Laiba Taseen

23-NTU-CS-1258

Embedded IOT Systems

Assignment-1

Question3

Task A:

#include <Arduino.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// --- Pin Setup ---

#define LED1 23

#define LED2 25

#define BTN_MODE 18

#define BTN_RESET 19

#define BUZZER 13

// --- Variables ---

int mode = 0;

unsigned long lastPress = 0;

const int debounceDelay = 300;

// --- Function: short beep for feedback ---

void beep() {

digitalWrite(BUZZER, HIGH);

delay(100);

digitalWrite(BUZZER, LOW);

}

// --- OLED display function ---

void showMode(const String &text) {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(10, 30);

display.println(text);

display.display();

}

// --- Reset button check function ---

bool resetPressed() {

if (digitalRead(BTN_RESET) == LOW && millis() - lastPress > debounceDelay) {

lastPress = millis();

return true;

}

return false;

}

// --- Turn everything OFF ---

void turnAllOff() {

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

digitalWrite(LED1, LOW);

digitalWrite(LED2, LOW);

digitalWrite(BUZZER, LOW);

showMode("Mode 0: Both OFF");

}

void setup() {

Serial.begin(115200);

Wire.begin(21, 22); // SDA=21, SCL=22

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println(F("OLED init failed"));

while (true);

}

pinMode(LED1, OUTPUT);

pinMode(LED2, OUTPUT);

pinMode(BUZZER, OUTPUT);

pinMode(BTN_MODE, INPUT_PULLUP);

pinMode(BTN_RESET, INPUT_PULLUP);

turnAllOff();

}

void loop() {

// --- Mode Change Button ---

if (digitalRead(BTN_MODE) == LOW && millis() - lastPress > debounceDelay) {

mode = (mode + 1) % 4;

lastPress = millis();

beep();

}

// --- Reset Button ---

if (resetPressed()) {

mode = 0;

beep();

turnAllOff();

return;

}

// --- LED Behavior ---

switch (mode) {

case 0: // Both OFF

turnAllOff();

break;

case 1: // Alternate blink

showMode("Mode 1: Alternate");

digitalWrite(LED1, HIGH);

digitalWrite(LED2, LOW);

for (int t = 0; t < 30; t++) {

if (resetPressed()) { mode = 0; turnAllOff(); return; }

delay(10);

}

digitalWrite(LED1, LOW);

digitalWrite(LED2, HIGH);

for (int t = 0; t < 30; t++) {

if (resetPressed()) { mode = 0; turnAllOff(); return; }

delay(10);

}

break;

case 2: // Both ON + Buzzer ON

showMode("Mode 2: Both ON");

digitalWrite(LED1, HIGH);

digitalWrite(LED2, HIGH);

digitalWrite(BUZZER, HIGH);

break;

case 3: // PWM fade

showMode("Mode 3: Fade");

digitalWrite(BUZZER, LOW);

// Attach PWM directly (Wokwi-compatible)

ledcAttach(LED1, 5000, 8);

ledcAttach(LED2, 5000, 8);

for (int i = 0; i <= 255; i++) {

if (resetPressed()) { mode = 0; turnAllOff(); return; }

ledcWrite(LED1, i);

ledcWrite(LED2, 255 - i);

delay(5);

}

for (int i = 255; i >= 0; i--) {

if (resetPressed()) { mode = 0; turnAllOff(); return; }

ledcWrite(LED1, i);

ledcWrite(LED2, 255 - i);

delay(5);

}

break;

}

}

Task B:

// Task B - Fixed for Wokwi/ESP32 (short vs long press)

#include <Arduino.h>

#include <Wire.h>

#include <Adafruit_GFX.h>

#include <Adafruit_SSD1306.h>

#define SCREEN_WIDTH 128

#define SCREEN_HEIGHT 64

Adafruit_SSD1306 display(SCREEN_WIDTH, SCREEN_HEIGHT, &Wire, -1);

// --- Pin Definitions ---

#define BUTTON_PIN 18

#define LED_PIN    4

#define BUZZER_PIN 13

// --- Variables ---

unsigned long pressStart = 0;

bool buttonPressed = false;

bool ledState = false;

const unsigned long LONG_PRESS_TIME = 1500; // 1.5 seconds

const unsigned long DEBOUNCE_MS = 50;

// debounce + long press control

unsigned long lastBounce = 0;

bool longHandled = false;

// --- Helper: Display message on OLED ---

void showMessage(const char* msg) {

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(SSD1306_WHITE);

display.setCursor(0, 20);

display.print(msg);

display.display();

}

void setup() {

Serial.begin(115200);

Wire.begin(21, 22);  // SDA=21, SCL=22

if (!display.begin(SSD1306_SWITCHCAPVCC, 0x3C)) {

Serial.println(F("SSD1306 allocation failed"));

for (;;);

}

display.clearDisplay();

display.display();

pinMode(BUTTON_PIN, INPUT_PULLUP);

pinMode(LED_PIN, OUTPUT);

pinMode(BUZZER_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

// --- Setup buzzer PWM (newer ESP32 cores support ledcAttach) ---

ledcAttach(BUZZER_PIN, 2000, 8); // pin, frequency, resolution (8-bit)

ledcWrite(BUZZER_PIN, 0);        // start silent

showMessage("Ready...");

}

void shortPressAction() {

ledState = !ledState;

digitalWrite(LED_PIN, ledState ? HIGH : LOW);

showMessage("Short Press:\nLED Toggled");

Serial.println("Short press");

}

void longPressAction() {

showMessage("Long Press:\nBuzzer Tone");

Serial.println("Long press");

// Play 500ms tone

ledcWrite(BUZZER_PIN, 200);  // 50% duty -> audible tone

delay(500);

ledcWrite(BUZZER_PIN, 0);    // stop tone

}

void loop() {

int reading = digitalRead(BUTTON_PIN);

// pressed

if (reading == LOW && !buttonPressed && (millis() - lastBounce) > DEBOUNCE_MS) {

buttonPressed = true;

pressStart = millis();

longHandled = false;

lastBounce = millis();

}

// held down

if (buttonPressed && !longHandled && (millis() - pressStart) >= LONG_PRESS_TIME) {

longPressAction();

longHandled = true;

}

// released

if (reading == HIGH && buttonPressed && (millis() - lastBounce) > DEBOUNCE_MS) {

unsigned long pressDuration = millis() - pressStart;

buttonPressed = false;

lastBounce = millis();

if (!longHandled && pressDuration < LONG_PRESS_TIME) {

shortPressAction();

}

longHandled = false;

}

delay(5);

}

Hand Sketch:

GitHub Link:

https://github.com/LaibaTaseen/Embedded-IOT-1258-/tree/master/Assignment1-1258