Project Title :

ESP32 Dual Switch - Dual LED using External Interrupts and Timer Debounce

Code:

/*

Project Title : ESP32 Dual Switch - Dual LED using External Interrupts and Timer Debounce

Author        : Laiba Taseen

Reg No          : 23-NTU-CS-1258

*/

#include <Arduino.h>

#define LED1_PIN  2

#define LED2_PIN  4

#define SW1_PIN   32

#define SW2_PIN   14

// Debounce time (milliseconds)

#define DEBOUNCE_MS  50

hw_timer_t *debounceTimer1 = NULL;

hw_timer_t *debounceTimer2 = NULL;

volatile bool sw1_pending = false;

volatile bool sw2_pending = false;

// Function declarations

void IRAM_ATTR sw1_isr();

void IRAM_ATTR sw2_isr();

void IRAM_ATTR debounceTimer1ISR();

void IRAM_ATTR debounceTimer2ISR();

void setup() {

Serial.begin(115200);

delay(100);

Serial.println("ESP32 Dual Switch Dual LED Project - Core v3.x Compatible");

pinMode(LED1_PIN, OUTPUT);

pinMode(LED2_PIN, OUTPUT);

digitalWrite(LED1_PIN, LOW);

digitalWrite(LED2_PIN, LOW);

pinMode(SW1_PIN, INPUT_PULLUP);

pinMode(SW2_PIN, INPUT_PULLUP);

debounceTimer1 = timerBegin(20000);

debounceTimer2 = timerBegin(20000);

// Attach ISRs

timerAttachInterrupt(debounceTimer1, debounceTimer1ISR);

timerAttachInterrupt(debounceTimer2, debounceTimer2ISR);

attachInterrupt(digitalPinToInterrupt(SW1_PIN), sw1_isr, FALLING);

attachInterrupt(digitalPinToInterrupt(SW2_PIN), sw2_isr, FALLING);

}

void loop() {

static unsigned long lastPrint = 0;

if (millis() - lastPrint > 2000) {

lastPrint = millis();

Serial.printf("LED1: %d | LED2: %d\n", digitalRead(LED1_PIN), digitalRead(LED2_PIN));

}

}

void IRAM_ATTR sw1_isr() {

detachInterrupt(digitalPinToInterrupt(SW1_PIN));

sw1_pending = true;

timerAlarm(debounceTimer1, DEBOUNCE_MS * 1000, false, 0);

}

void IRAM_ATTR sw2_isr() {

detachInterrupt(digitalPinToInterrupt(SW2_PIN));

sw2_pending = true;

timerAlarm(debounceTimer2, DEBOUNCE_MS * 1000, false, 0);

}

// Timer ISR for Switch 1

void IRAM_ATTR debounceTimer1ISR() {

if (sw1_pending && digitalRead(SW1_PIN) == LOW) {

digitalWrite(LED1_PIN, !digitalRead(LED1_PIN));

}

sw1_pending = false;

attachInterrupt(digitalPinToInterrupt(SW1_PIN), sw1_isr, FALLING);

}

// Timer ISR for Switch 2

void IRAM_ATTR debounceTimer2ISR() {

if (sw2_pending && digitalRead(SW2_PIN) == LOW) {

digitalWrite(LED2_PIN, !digitalRead(LED2_PIN));

}

sw2_pending = false;

attachInterrupt(digitalPinToInterrupt(SW2_PIN), sw2_isr, FALLING);

}

Output Screenshot:

Wokwi Project Link:

https://wokwi.com/projects/443984382660231169