Title:
MQTT Data Subscription and Debug Display using Node-RED

Objective:
To subscribe DHT22 temperature data published by ESP32 using MQTT and display it in Node-RED Debug window.

Tools / Software Used:

- ESP32 (Simulator in Wokwi / VS Code)

- DHT22 sensor

- Mosquitto MQTT Broker (running on port 1883)

- Node-RED (http://localhost:1880)

Procedure (Step-by-Step):

- Start Mosquitto Broker:
Open terminal / command prompt and run:

- mosquitto -v

Output should confirm:

mosquitto running

- Run ESP32 Code (Publisher):

  - Make sure ESP32 code has correct broker IP:

  - const char* mqtt_server = "";

  - ESP32 publishes temperature to topic:

  - home/lab1/temp

  - Serial monitor shows:

  - MQTT connected

  - Publishing temperature: 26.5

- Start Node-RED:

- node-red

Open browser: http://localhost:1880

- Add MQTT-in Node:

  - Drag MQTT-in node

  - Configure:

    - Server: localhost

    - Port: 1883

    - Topic: home/lab1/temp

    - QoS: 0

    - Action: Subscribe to single topic

  - Click Done

- Add Debug Node:

  - Drag Debug node

  - Output: msg.payload

  - Connect: MQTT-in → Debug

- Deploy Flow:

  - Click Deploy

  - Open Debug tab (right panel)

  - You should see temperature values like:

  - 26.4

  - 27.0

- Screenshots:

  - Mosquitto running terminal

  - ESP32 serial output (Wokwi)

  - Node-RED flow (MQTT-in → Debug)

Conclusion:
ESP32 successfully published DHT22 temperature data using MQTT protocol. Node-RED subscribed to the topic home/lab1/temp through a local Mosquitto broker. The received data was displayed in the Debug window, demonstrating successful IoT data monitoring.